2304 – 119 Avenue NE

Edmonton, Alberta

T6S 1B3

April 13, 2026

File No. 3343 – 1

LAUNCH HOMES INC.

9310 – 62 Avenue NW

Edmonton, Alberta

T6E 0C9

Re:	Foundation Soil Inspection

Lot 7, Block 10, Plan 252 1945

26 Cuddie Drive

Job# CBR-6223

Keswick Subdivision

Edmonton, Alberta

As requested, J.R. PAINE ASSOCIATES LTD., (JRP), has conducted an inspection of the above noted excavation on April 8, 2026.  The purpose of this inspection was to verify that the site soil bearing capacity and expected settlement are adequate to support typical house loads, and where they are not adequate, to recommend a foundation design to address these bearing and settlement issues.  It is assumed that this house is designed and will be constructed in accordance with the Alberta Building Code, except where superseded by this letter.

At the time of inspection, the excavation was at footing grade and strip footing forms with a footing width of approximately 450 millimetres and depth of 150 millimetres had been placed. Cuts of approximately 1.5 to 2.9 m below the adjacent ground surface were noted.

The soil encountered throughout the excavation floor was identified as moist, brown, silty, clay till.  The clay material was a native deposit of medium plasticity with a very stiff consistency and contained traces of coal, oxides and gravel.

The soil conditions at this site were considered suitable for the construction of a standard house foundation. The strip footing size, as placed, was considered adequate for this residence. Spread footings may be designed utilizing a factored bearing resistance of 120 kpa (2500 psf) with a geotechnical resistance factor of 0.5.

It appears the building contractor utilized a standard footing form for the attached garage.  The strip footing size, as constructed, was also considered adequate for this garage.  Also, a minimum cover of 2.0 metres for frost protection should be provided for non-continuously heated structures or exterior isolated footings.  The frost cover can be reduced to 1.5 metres for continuously heated structures.  As an option, frost cover can be obtained by the use of rigid insulation.  Insulation details can be supplied upon request.

During cold weather construction, it is essential that all interior fill and load bearing material remain as frost free as possible.  In order to minimize the potential frost heave problems, the interior of the building must be heated as soon as the walls have been poured and the sub-floor is placed.  The period in which the excavation is left open to freezing conditions should be as short as possible.  There must be no delays from the time of excavation to the placement of the temporary heat system.  Recommended cold weather construction practices, with respect to hoarding and heating of the forms and the fresh concrete, should be followed.

We trust this information is considered satisfactory.  Should you have any questions, please contact our office.

Yours truly,

J. R. Paine & Associates Ltd.

Andrew R. Klein, B. Sc.

Reviewed by,

Rick Evans, P. Eng.

APEGA MEMBER #: 49679

H:\DATA 2026\00 Housing 2026\3343-1 Bedrock Homes Ltd\h38937bed.docx