2304-119 Avenue NE

Edmonton, Alberta

T6S 1B3

March 12, 2026

File No. 4460 – 1

Oasis Engineering GROUP Ltd.

202, 10335 - 178 Street NW

Edmonton, Alberta

T5S 1S2

Re:		Foundation Soils Inspection

Lot 11 and 12 Block 28, Plan 242 2337

18115 and 18117 – 74 Street NW

Crystallina Nera Subdivision

Edmonton, Alberta

As requested, J. R. Paine & Associates Ltd., (JRP), has conducted an inspection of the above noted excavation on March 10, 2026.  The purpose of this inspection was to verify that the site soil bearing capacity and expected settlement are adequate to support typical house loads, and where they are not adequate, to recommend a foundation design to address these bearing and settlement issues.  It is assumed that this duplex is designed and will be constructed in accordance with the Alberta Building Code, except where superseded by this letter.

At the time of inspection, the excavation was at footing grade and was advanced by a backhoe.  Cuts of approximately 1.5 to 3.2 m below the adjacent ground surface were noted in the house area.  In addition, the excavation had been extended into the garage footing areas, with the bottom of that excavation at the same elevation as the house excavation floor.

The soil encountered to a depth of approximately 0.8 m throughout the excavation floor was identified as moist, brown, silty, clay fill.  The clay fill material was of medium plasticity with a very stiff consistency and contained traces of organics.  Below the clay fill material the soil encountered was identified as moist, brown, silty, clay.  The clay material was a native deposit of medium plasticity with a stiff consistency.  This lot was part of an engineered fill program monitored and tested by our firm.

The soil conditions at this site were considered suitable for the construction of a standard house foundation.  A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended for this residence.  Spread footings may be designed utilizing a factored bearing resistance of 140 kilopascals (2900 pounds per square foot) with a geotechnical resistance factor of 0.5.

It is understood the building contractor wishes to utilize a standard footing foundation for the attached garage.  A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended for the garage.

A representative sample of soil was gathered from the excavation site and tested to estimate the concentration of soluble soil sulphates present in the soil.  The test revealed a severe concentration of sulphates present in the sample.  Based on C.S.A. A23.1-24, class of exposure S-2 should be applied to the design requirements for concrete in contact with the soil and susceptible to sulphate degradation.  The class S-2 exposure requires Type HS cement and a minimum 56 day concrete strength of 32 MPa, as well as other requirements given in the noted CSA guideline.

During cold weather construction, it is essential that all interior fill and load bearing material remain as frost free as possible.  In order to minimize the potential frost heave problems, the interior of the building must be heated as soon as the walls have been poured and the sub-floor is placed. The period in which the excavation is left open to freezing conditions should be as short as possible. There must be no delays from the time of excavation to the placement of the temporary heat system.  Recommended cold weather construction practices, with respect to hoarding and heating of the forms and the fresh concrete, should be followed.

We trust this information is considered satisfactory.  Should you have any questions, please contact our office.

Yours truly,

J. R. Paine & Associates Ltd.

Andrew R. Klein, B. Sc.

Reviewed by,

Al Lang, P. Eng.

APEGA Member # 46743

H:\DATA 2026\00 Housing 2026\4460-1 Oasis Engineering Group Ltd\h38764oas.docx