J.R. Paine & Associates Ltd.

Edmonton office block placeholder

Exact office address lines to be confirmed.

April 15, 2026

File No.: 5478 - 1

VICTORY HOMES LTD.

1665 - 91 Street SW

Edmonton, Alberta

T6X 0W8

Re: Foundation Soil Inspection

Lot 110, Block 17, Plan 252 2250

607 - 30 Street SW

Alces Subdivision

Edmonton, Alberta

As requested, J.R. Paine & Associates Ltd., (JRP), has conducted an inspection of the above noted excavation on January 28, 2026.  The purpose of this inspection was to verify that the site soil bearing capacity and expected settlement are adequate to support typical house loads, and where they are not adequate, to recommend a foundation design to address these bearing and settlement issues.  It is assumed that this house is designed and will be constructed in accordance with the Alberta Building Code, except where superseded by this letter.

At the time of inspection, the excavation was at footing grade and was advanced by a backhoe. Cuts of approximately 1.7 to 1.9 m below the adjacent ground surface were noted in the house area. In addition, the excavation had been extended into the garage footing areas, with the bottom of that excavation at the same elevation as the house excavation floor.

The soil encountered throughout the excavation floor was identified as moist, brown and grey, clay. The clay was a native deposit of medium plasticity with a stiff consistency and featured traces of oxides.

The soil conditions at this site were considered adequate for the construction of a standard house footing foundation. A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended. Spread footings may be designed utilizing a factored bearing resistance of 140 kilopascals (2900 pounds per square foot) with a geotechnical resistance factor of 0.5.

It is understood the building contractor wishes to utilize a standard footing foundation for the attached garage. A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended for the garage. Also, a minimum cover of 2.0 m for frost protection should be provided for non-continuously heated structures or exterior isolated footings. The frost cover can be reduced to 1.5 m for continuously heated structures. As an option, frost cover can be obtained by the use of exterior rigid insulation. Insulation details can be supplied upon request.

During cold weather construction, it is essential that all interior fill and load bearing material remain as frost free as possible. In order to minimize the potential frost heave problems, the interior of the building must be heated as soon as the walls have been poured and the sub- floor is placed. The period in which the excavation is left open to freezing conditions should be as short as possible. There must be no delays from the time of excavation to the placement of the temporary heat system. Recommended cold weather construction practices, with respect to hoarding and heating of the forms and the fresh concrete, should be followed.

We trust this information is considered satisfactory for your present requirements.

J.R. Paine & Associates Ltd.

Prepared by: Doug Parth, E.I.T.

Reviewed by: Scott MacFarlane, P.Eng.

Member No.: [registry pending]

[Engineer stamp placeholder: scott-macfarlane-stamp]

[Permit-to-practice signing area placeholder]