2304 – 119 Avenue NE

Edmonton, Alberta

T6S 1B3

April 7, 2026

File No. 4460 – 1

OASIS ENGINEERING GROUP LTD.

#202, 10335 – 178 Street NW 
Edmonton, Alberta
T5S 1S2

Re:	Foundation Soil Inspection

Lot 3, Block 15, Plan 252 1961

9789 Elves Place

Edgemont Subdivision

Edmonton, Alberta

As requested, J.R. Paine & Associates Ltd., (JRP), has conducted an inspection of the above noted excavation on April 1, 2026.  The purpose of this inspection was to verify that the site soil bearing capacity and expected settlement are adequate to support typical house loads, and where they are not adequate, to recommend a foundation design to address these bearing and settlement issues.  It is assumed that this house is designed and will be constructed in accordance with the Alberta Building Code, except where superseded by this letter.

At the time of inspection, the excavation was at footing grade and was advanced by a backhoe.  Cuts of approximately 1.5 m to 1.9 m below the adjacent ground surface were noted.  The excavation was extended into the attached garage area at the front and was made level with the main house basement excavation.  Loose materials were noted across the excavation floor.

The soil encountered throughout the excavation floor was identified as a native deposit of brown and grey, silty, very sandy clay.  The native clay encountered was considered low to medium plastic, moist, stiff to friable, and contained oxidation.

The soil conditions at this site were considered adequate to support a standard house footing foundation from the strength and settlement viewpoints.  A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended for the perimeter strip footing.  The interior spread footings may be designed utilizing a factored bearing resistance of 140 kPa, which included a geotechnical resistance factor of 0.5.

It is understood the building contractor wishes to utilize a standard footing foundation for the attached garage.  A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended for the garage.

During cold weather construction, it is essential that all interior fill and load bearing material remain as frost free as possible.  In order to minimize the potential frost heave problems, the interior of the building must be heated as soon as the walls have been poured and the sub-floor is placed.  The period in which the excavation is left open to freezing conditions should be as short as possible.  There must be no delays from the time of excavation to the placement of the temporary heat system.  Recommended cold weather construction practices, with respect to hoarding and heating of the forms and the fresh concrete, should be followed.

A representative sample of soil was gathered from the excavation site and tested to estimate the concentration of soluble soil sulphate present in the soil.  The test revealed a negligible concentration of sulphate was present and based on C.S.A. A23.1-24, Type GU cement may be used for all concrete in contact with the soil.

We trust this information is considered satisfactory.  Should you have any questions, please contact our office.

Yours truly,

J.R. Paine & Associates Ltd.

ID # 76087

John Tsoi, P. Eng.

H:\DATA 2026\00 Housing 2026\4460-1 Oasis Engineering Group Ltd\h38905oas.docx