2304-119 Avenue NE

Edmonton, Alberta

T6S 1B3

April 9, 2026

File No. 4460 – 1

Oasis Engineering GROUP Ltd.

202, 10335 - 178 Street NW

Edmonton, Alberta

T5S 1S2

Re:		Foundation Soils Inspection

Lots 92 & 93, Block 21, Plan 242 2238

2222 & 2224 Chokecherry Close SW

Orchards Subdivision

Edmonton, Alberta

As requested, J. R. Paine & Associates Ltd., (JRP), has conducted an inspection of the above noted excavation on April 8, 2026.  The purpose of this inspection was to verify that the site soil bearing capacity and expected settlement are adequate to support typical house loads, and where they are not adequate, to recommend a foundation design to address these bearing and settlement issues. It is assumed that this house is designed and will be constructed in accordance with the Alberta Building Code, except where superseded by this letter.

At the time of inspection, the excavation was at footing grade and was advanced by a backhoe.  Cuts of approximately 1.0 to 1.4 m below the adjacent ground surface were noted in the house area.  In addition, the excavation had been extended into the garage footing areas, with the bottom of that excavation at the same elevation as the house excavation floor. At the time of inspection, as oversized service trench was observed in the front of the excavation.

The soil encountered throughout the excavation floor was identified as moist, brown/grey, silty, sandy clay till. The clay till material was a native deposit of medium plasticity with a very stiff consistency and contained traces of oxides, coal, and gravel.

Due to an oversized cut in the service trench, the front garage footing should be reinforced with one row of 2-10M bars spread evenly throughout the footing.  The front garage wall should be reinforced with four rows of 2-10M bars, spread evenly throughout the wall.

The soil conditions at this site would then be considered adequate for the construction of a standard house footing foundation. A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended. Spread footings may be designed utilizing a factored bearing resistance of 140 kPa (2900 psf) with a geotechnical resistance factor of 0.5.

It is understood the building contractor wishes to utilize a standard footing foundation for the attached garage. A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended for the garage.

During cold weather construction, it is essential that all interior fill and load bearing material remain as frost free as possible.  In order to minimize the potential frost heave problems, the interior of the building must be heated as soon as the walls have been poured and the sub-floor is placed. The period in which the excavation is left open to freezing conditions should be as short as possible. There must be no delays from the time of excavation to the placement of the temporary heat system.  Recommended cold weather construction practices, with respect to hoarding and heating of the forms and the fresh concrete, should be followed.

We trust this information is considered satisfactory.  Should you have any questions, please contact our office.

Yours truly,

J. R. Paine & Associates Ltd.

Muhammad Huzaifa, E.I.T.

Reviewed by,

Scott MacFarlane, P.Eng.

APEGA Member #: 89667

H:\DATA 2026\00 Housing 2026\4460-1 Oasis Engineering Group Ltd\h38935oas.docx