2304 – 119 Avenue NE

Edmonton, Alberta

T6S 1B3

April 7, 2026

File No. 3343 – 1

BEDROCK HOMES LTD.

1253 – 91 Street SW

Edmonton, Alberta

T6X 1E9

Re:	Foundation Soil Inspection

Lot 98, Block 1, Plan 262 0213

17240 – 5 Street NW

Job# MAR - 2 - 036449

Marquis Subdivision

Edmonton, Alberta

As requested, J.R. PAINE ASSOCIATES LTD., (JRP), has conducted an inspection of the above noted excavation on April 1, 2026.  The purpose of this inspection was to verify that the site soil bearing capacity and expected settlement are adequate to support typical house loads, and where they are not adequate, to recommend a foundation design to address these bearing and settlement issues.  It is assumed that this house is designed and will be constructed in accordance with the Alberta Building Code, except where superseded by this letter.

At the time of inspection, frost was encountered throughout most of the excavation floor to a depth of approximately 150 mm. The excavation was at footing grade and was advanced by a backhoe. Cuts of approximately 2.0 to 2.1 m below the adjacent ground surface were noted in the house area. In addition, the excavation had been extended into the garage footing areas, with the bottom of that excavation at the same elevation as the house excavation floor. Surficial free water/ saturated material was noted on the right portion of the excavation.

The soil encountered throughout the excavation floor was identified as moist, brown/grey, silty, sandy to very sandy till like clay. The till like clay material was a native deposit of medium plasticity with a firm to stiff consistency and featured traces amounts of coal, oxides and gravel.

Due to the frost, the foundation footings should be reinforced with one row of 2-10M bars spread evenly throughout the footing.  The foundation walls should be reinforced with four rows of 2-10M bars, spread evenly throughout the wall. The water/saturated/water softened material should be adequately dried or removed from underneath the footing areas prior to pouring. The excavated material can be accounted for by a thickened footing. Care must also be taken to place the weeping tile level, with no bumps or sags and maintain positive drainage.

The soil conditions at this site would then be considered adequate for the construction of a standard house footing foundation.  A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended. Spread footings may be designed utilizing a factored bearing resistance of 140 kPa (2900 psf) with a geotechnical resistance factor of 0.5.

It is understood the building contractor wishes to utilize a standard footing foundation for the attached garage.  A minimum strip footing width of 450 mm and a minimum depth of 150 mm are recommended for the garage.  Also, a minimum cover of 2.0 m for frost protection should be provided for non-continuously heated structures or exterior isolated footings.  The frost cover can be reduced to 1.5 m for continuously heated structures.  As an option, frost cover can be obtained by the use of exterior rigid insulation.  Insulation details can be supplied upon request.

During cold weather construction, it is essential that all interior fill and load bearing material remain as frost free as possible.  In order to minimize the potential frost heave problems, the interior of the building must be heated as soon as the walls have been poured and the sub-floor is placed.  The period in which the excavation is left open to freezing conditions should be as short as possible.  There must be no delays from the time of excavation to the placement of the temporary heat system.  Recommended cold weather construction practices, with respect to hoarding and heating of the forms and the fresh concrete, should be followed.

We trust this information is considered satisfactory.  Should you have any questions, please contact our office.

Yours truly,

J. R. Paine & Associates Ltd.

Darren Wang, E.I.T.

Reviewed by,

Scott MacFarlane, P.Eng.

APEGA Member #: 89667

H:\DATA 2026\00 Housing 2026\3343-1 Bedrock Homes Ltd\h38894bed.docx