2304-119 Avenue NE

Edmonton, Alberta

T6S 1B3

April 7, 2026

File No. 4460 – 1

Oasis Engineering GROUP Ltd.

202, 10335 - 178 Street NW

Edmonton, Alberta

T5S 1S2

Re:		Foundation Soils Inspection

Lot 16, Block 12, Plan 252 1068

1433 Osprey Court NW

Hawks Ridge Subdivision

Edmonton, Alberta

As requested, J. R. Paine & Associates Ltd., (JRP), has conducted an inspection of the above noted excavation on March 24, 2026.  The purpose of this inspection was to verify that the site soil bearing capacity and expected settlement are adequate to support typical house loads, and where they are not adequate, to recommend a foundation design to address these bearing and settlement issues.  It is assumed that this house is designed and will be constructed in accordance with the Alberta Building Code, except where superseded by this letter.

At the time of inspection, the excavation was at footing grade and was advanced by a backhoe.  Cuts of approximately 1.9 to 2.4 m below the adjacent ground surface were noted in the house area.  In addition, the excavation had been extended into the garage footing areas, with the bottom of that excavation at the same elevation as the house excavation floor.

The soil encountered throughout the excavation floor was identified as moist, brown and grey, silty, sandy clay.  The clay material was a native deposit of medium plasticity with a firm consistency and featured wet sand lenses.

At the time of inspection, free water was noted pooling in the augur holes approximately 0.2 metres below the footing elevation. Due to the infiltrating water in the augur holes, the house drainage should be upgraded to include a washed rock slab base and interior as well as exterior weeping tile with two laterals. A drawing depicting the recommended drainage measures is attached.

The soil conditions at this site would then be considered suitable for the construction of a modified house foundation. A minimum strip footing width of 600 mm and a minimum depth of 200 mm are recommended. Spread footings may be designed utilizing a factored bearing resistance of 100 kilopascals (2000 pounds per square foot) with a geotechnical resistance factor of 0.5.

It is understood the building contractor wishes to utilize a modified footing foundation for the attached garage.  A minimum strip footing width of 600 mm and a minimum depth of 200 mm are recommended for the garage.

During cold weather construction, it is essential that all interior fill and load bearing material remain as frost free as possible.  In order to minimize the potential frost heave problems, the interior of the building must be heated as soon as the walls have been poured and the sub-floor is placed. The period in which the excavation is left open to freezing conditions should be as short as possible. There must be no delays from the time of excavation to the placement of the temporary heat system.  Recommended cold weather construction practices, with respect to hoarding and heating of the forms and the fresh concrete, should be followed.

We trust this information is considered satisfactory.  Should you have any questions, please contact our office.

Yours truly,

J. R. Paine & Associates Ltd.

Doug Parth, E.I.T.

Reviewed by,

Scott MacFarlane, P.Eng.

APEGA Member #: 89667

H:\DATA 2026\00 Housing 2026\4460-1 Oasis Engineering Group Ltd\h38862oas.docx